Shanghai Goodview Electronics Technology Co. Ltd.

65 inch Commercial LCD Monitor

Model:M65SAL

Revision History

| Part number | Version | Publish date | Written by | Auditing | approval | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
| 1158701 | GV-M65SAL-GGS20180109 | 2018\1\9 |  |  |  |  |

Note：All right reserved by Goodview.Please contact with us for the latest updates.

Product Specifications

| Display |  |  |  |
| --- | --- | --- | --- |
| Panel | 64.53 inches LED backlight |  |  |
| Active Area (mm) | 803.5（H）×1428.5（V） |  |  |
| Aspect Ratio | 16:9 |  |  |
| Resolution | 3840×2160 |  |  |
| Brightness | 350 cd/m2 |  |  |
| Contrast Ratio | 960:1 |  |  |
| Viewing Angle (CR>10) | All view angle ( R/L 158 (Min.), U/D 158 (Min.)) |  |  |
| Display Colors | 10bit, 1.07 Billion |  |  |
| Color temperature | TBD |  |  |
| Back light bulb Life | 30000 hours |  |  |
| Response time | 8ms |  |  |
| System configuration |  |  |  |
| Operating system | Android 5.0.1 |  |  |
| CPU | ARM A53×4 1.4GHz |  |  |
| GPU | ARM Mali450 MP4(4+2) 450MHz |  |  |
| Memory | 2G |  |  |
| Storage | 16G |  |  |
| External ports |  |  |  |
| Signal ports | HDMI(1.3) IN×2 VGA IN×1 VGA Audio IN×1 DP IN×1 AV ×1 YPBPR×1 AV OUT×1 Audio OUT×1 |  |  |  |
| Network ports | Wired network: LAN×1（RJ45 10/100M Ethernet） wireless network: WiFi×1（802.11bgn） |  |  |  |
| Storage extension | Micro SD Card ×1 |  |  |  |
| Other ports | AC Switch×1 AC IN×1 USB 3.0 IN×2 Touch OUT USB（TypeB）×1 IR IN×1 (3.5mm JACK) RS232×1 (DB9) SPDIF×1 |  |  |  |
| Multimedia |  |  |  |
| picture | File extension name | *.jpg, *.bmp, *.png |  |
|  | The encoder | Standard RGB |  |
|  | Maximum image resolution | Baseline : JPG : 6000 x 6000 GIF : 1920 x 1080 BMP : 1920 x 1080 PNG : 1920 x 1080 |  |
|  | Maximum filename length | 128 bytes |  |
| video | File extension name | *.mpg, *.avi, *.mp4, *.mkv, *flv, *.mov, , *.ts, |  |
|  | Coding and code rate | MPEG-1 bitrate<20Mbps MPEG-2 bitrate<40Mbps MPEG-4 bitrate<30Mbps H.264 bitrate<30Mbps AVC bitrate<30Mbps H263 bitrate<30Mbps XVID bitrate<30Mbps |  |
| audio | File extension name | *.mp3,AAC, wav |  |
|  | The encoder | MPEG Audio, PCM |  |
| text | File extension name | *.txt |  |
|  | Encoder format | ANSI、 |  |
| Play mode | Horizontal screen, Vertical screen, full screen and split screen play |  |  |
| Program transmission mode | USB、LAN、Wan |  |  |
| Other parameters |  |  |  |
| Product size (mm) | 891.95(H)×1516.5(V)×110.8(D) |  |  |
| Package size（mm） | 1200（H）×1755（V）×345（D） |  |  |
| VESA（mm） | 400×600 & 400×400 （Screw type M8×25mm） |  |  |
| Screen hanging mode | landscape、portrait |  |  |
| Menu language | Chinese、English |  |  |
| Machine color | black |  |  |
| Weight | net：45KG (TBD) |  | gross：45KG (TBD) |
| Power Consumption | 130W (Max) |  |  |
| Speaker | stereo 2×10W 8Ω |  |  |
| Smart volume | time-sharing volume |  |  |
| Watchdog | N/A |  |  |
| Timing function | Multi-time timing power on/off, port timing automatic switch |  |  |
| Management software | GTV Information Distribution System |  |  |
| Power supply | AC~100-240V (50/60Hz 3 Hz ) |  |  |
| Working environment | 0〜40℃ 10% 〜 85%RH No condensation |  |  |
| Storage environment | -10〜60℃ 10% 〜 75%RH No condensation |  |  |
| Accessories | 1. Warranty card * 1. 2. Certificate of qualification 3. Quick start guide x 1. 4. AC power line x 1 (national standard) 5. 2.4G WiFi antenna x 1 root. 6. Remote control * 1. 7. AAA battery x 2Pcs. 8.Safety and environmental considerations *1 |  |  |
| Certificates | CCC/FCC |  |  |

Schematic Drawing

- Product Images

AC SW

AC IN

Packaging

unit：mm

Note：All right reserved by Goodview.Please contact with us for the latest updates.