Shanghai Goodview Electronics Technology Co. Ltd.

75 inch Commercial LCD Monitor

Model:M75STL

Revision History

| Part number | Version | Publish date | Written by | Auditing | approval | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
| 1374601 | GV-M75STL-GGS20180109 | 2018\1\9 |  |  |  |  |

Note：All right reserved by Goodview.Please contact with us for the latest updates.

Product Specifications

| Display |  |  |  |  |
| --- | --- | --- | --- | --- |
| Panel |  | 75 inches LED backlight |  |  |
| Active Area (mm) |  | 928.26（H）×1650.24（V） |  |  |
| Aspect Ratio |  | 16:9 |  |  |
| Resolution |  | 3840×2160 |  |  |
| Brightness |  | 430 cd/m2 |  |  |
| Contrast Ratio |  | 1000:1 |  |  |
| Viewing Angle (CR>10) |  | All view angle ( R/L 89 (Min.), U/D 89 (Min.)) |  |  |
| Display Colors |  | 10bit, 1.07 Billion |  |  |
| Color temperature |  | TBD |  |  |
| Back light bulb Life |  | 30000 hours |  |  |
| Response time |  | 8ms |  |  |
| System configuration |  |  |  |  |
| Operating system |  | Android 5.0.1 |  |  |
| CPU |  | ARM A53×4 1.4GHz |  |  |
| GPU |  | ARM Mali450 MP4(4+2) 450MHz |  |  |
| Memory |  | 2G |  |  |
| Storage |  | 16G |  |  |
| Touch |  |  |  |  |  |
| Touch type |  | Infrared touch |  |  |  |
| Touch Points |  | 20 points |  |  |  |
| Interpolation resolution |  | 32767×32767 |  |  |  |
| Touch accuracy |  | ±2mm(More than 90% of the touch area) |  |  |  |
| Response time |  | <15ms |  |  |  |
| Enter mode |  | Finger, stylus, opaque objects |  |  |  |
| Smallest touch objects | Single | 6mm |  |  |
|  | Multipoint | 8mm |  |  |
| External ports |  |  |  |  |
| Signal ports |  | HDMI(1.3) IN×2 VGA IN×1 VGA Audio IN×1 DP IN×1 AV ×1 YPBPR×1 AV OUT×1 Audio OUT×1 |  |  |  |
| Network ports |  | Wired network: LAN×1（RJ45 10/100M Ethernet） wireless network: WiFi×1（802.11bgn） |  |  |  |
| Storage extension |  | Micro SD Card ×1 |  |  |  |
| Other ports |  | AC Switch×1 AC IN×1 USB 3.0 IN×2 Touch OUT USB（TypeB）×1 IR IN×1 (3.5mm JACK) RS232×1 (DB9) SPDIF×1 |  |  |  |
| Multimedia |  |  |  |  |
| picture |  | File extension name | *.jpg, *.bmp, *.png |  |
|  |  | The encoder | Standard RGB |  |
|  |  | Maximum image resolution | Baseline : JPG : 6000 x 6000 GIF : 1920 x 1080 BMP : 1920 x 1080 PNG : 1920 x 1 |  |
|  |  | Maximum filename length | 128 bytes |  |
| video |  | File extension name | *.mpg, *.avi, *.mp4, *.mkv, *flv, *.mov, , *.ts, |  |
|  |  | Coding and code rate | MPEG-1 bitrate<20Mbps MPEG-2 bitrate<40Mbps MPEG-4 bitrate<30Mbps H.264 bitrate<30Mbps AVC bitrate<30Mbps H263 bitrate<30Mbps XVID bitrate<30Mbps |  |
| audio |  | File extension name | *.mp3,AAC, wav |  |
|  |  | The encoder | MPEG Audio, PCM |  |
| text |  | File extension name | *.txt |  |
|  |  | Encoder format | ANSI、 |  |
| Play mode |  | Horizontal screen, Vertical screen, full screen and split screen play |  |  |
| Program transmission mode |  | USB、LAN、Wan |  |  |
| Other parameters |  |  |  |  |
| Product size (mm) |  | 1045.23(H)×1767.16(V)×98.8(D) |  |  |
| Package size（mm） |  | 1351（H）×2051（V）×394（D） |  |  |
| VESA（mm） |  | 400×600 （Screw type M8×25mm） |  |  |
| Screen hanging mode |  | landscape、portrait |  |  |
| Menu language |  | Chinese、English |  |  |
| Machine color |  | black |  |  |
| Weight |  | net：54KG (TBD) |  | gross：69KG (TBD) |
| Power Consumption |  | 260W (Max) |  |  |
| Speaker |  | stereo 2×10W 8Ω |  |  |
| Smart volume |  | time-sharing volume |  |  |
| Watchdog |  | N/A |  |  |
| Timing function |  | Multi-time timing power on/off, port timing automatic switch |  |  |
| Management software |  | GTV Information Distribution System |  |  |
| Power supply |  | AC~100-240V (50/60Hz 3 Hz ) |  |  |
| Working environment |  | 0〜40℃ 10% 〜 85%RH No condensation |  |  |
| Storage environment |  | -10〜60℃ 10% 〜 75%RH No condensation |  |  |
| Accessories |  | 1. Warranty card * 1. 2. Certificate of qualification 3. Quick start guide x 1. 4. AC power line x 1 (national standard) 5. 2.4G WiFi antenna x 1 root. 6. Remote control * 1. 7. AAA battery x 2Pcs. - 8.Safety and environmental considerations *1 |  |  |
| Certificates |  | / |  |  |

Schematic Drawing

- Product Images

AC IN

AC SW

Packaging

unit：mm

Note：All right reserved by Goodview.Please contact with us for the latest updates.